### Summary

The report titled **"Creation of an Integrated System for Countering Enemy UAVs in the Area of Responsibility of a Combined Arms Army"** by the Electronic Warfare (EW) Service of the 49th Army (2024) details the development, deployment, and effectiveness of a comprehensive system designed to detect, suppress, and destroy enemy unmanned aerial vehicles (UAVs), particularly commercial reconnaissance and strike UAVs, including FPV (First-Person View) drones.

---

### Background and Rationale

- The enemy has significantly increased the use of commercial reconnaissance and strike UAVs, including FPV drones.

- Mobilized combined arms army units lacked standard EW forces and means.

- Decision was made to involve sponsors and develop a **unified counter-UAV system** consolidating detection, electronic suppression of control and video channels, and destruction of UAVs and their control points.

- The system is built at the battalion, brigade (regiment), and army levels, integrated into a centralized army UAV countermeasure command.

---

### System Development and Sponsorship

- The **Kherson regional administration** sponsored the creation of the **“KRAB” group** (Complex Radio-Electronic Anti-Drone Combat) based on the 512th EW detachment of the 49th Army.

- Equipment was procured, and the group trained successfully.

- Since May 14, 2024, KRAB has operated initially in a motorized rifle battalion’s defense zone, then at the regiment level, and currently covers the entire 49th Army area.

- The system integrates UAV detection posts of first-echelon battalions, EW posts, reconnaissance-strike UAV calculations, air defense (AD), artillery units, and army-level AD and artillery formations.

---

### System Architecture and Components

**At the Motorized Rifle Battalion Level:**

- Detection posts include:

- **SDR spectrum analyzer receiver (HackRF)** with a directional antenna ("masterok") connected to a computer running SDR++, enabling identification of enemy FPV control frequencies and providing warnings.

- **Automatic video channel scanner ("Owl")** scanning 4.8–6.2 GHz to intercept analog FPV video signals up to 25 km, revealing drone launch sites and flight paths.

- Computers with internet access for data streaming and situational awareness.

- Communication tools: DMR radios and TA-57 radios for links with EW posts, UAVs, and regimental detection posts.

- EW assets in motorized rifle companies include radio jammers (GROZA-YUB, Carlson, Mgla-10, J2, etc.) targeting enemy FPV control and video transmission channels.

- Use of captured drones with directional antennas or high-power video transmitters (5-10 W) to suppress enemy UAV video links (typically 2.5 W omnidirectional).

**At the Brigade (Regiment) Level:**

- Detection posts intercept analog video signals of FPV drones, "Baba Yaga," and reconnaissance UAVs with analog transmitters at 1.2 GHz and 5.8 GHz using directional antennas and rotating mounts.

- Digital video cameras with thermal imaging and high zoom capabilities are deployed.

- Posts are connected via wired internet or Starlink terminals for real-time online data sharing.

- Integration with battalion detection posts, EW units, UAV units, artillery, and air defense elements.

- Army-level system connects brigade/regiment detection posts and units of air defense, artillery, EW, electronic reconnaissance (ER), and UAVs.

---

### Equipment Overview

| Category                     | Equipment / System                   | Functionality                                   |

|------------------------------|-----------------------------------|------------------------------------------------|

| Spectrum Analyzer Receiver    | HackRF with masterok antenna      | Detects FPV control frequencies                 |

| Video Channel Scanner         | "Owl" (4.8–6.2 GHz)               | Intercepts analog FPV video signals              |

| Communication Tools           | DMR radios, TA-57 radios          | Ensures connectivity between posts and units   |

| Radio Jammers                 | GROZA-YUB, Carlson, Mgla-10, J2  | Suppresses UAV control and video channels       |

| High-Power Video Transmitters | 2.5 W and 5 W transmitters        | Used for video jamming                           |

| UAV Types                    | Hunter UAV, Matrix 300 RTK, UAV with thermal imaging | Reconnaissance and strike roles                  |

| Automated EW Stations         | R-330JS, Prostor-403              | Army/brigade standard EW means                   |

---

### Performance and Effectiveness

| Metric                                   | Before KRAB System | After KRAB Implementation |

|------------------------------------------|-------------------|---------------------------|

| Detection Efficiency of Enemy FPV Drones | Up to 40%         | Over 95%                  |

| Destruction via Radio Jamming             | Up to 30%         | 70-80%                    |

---

### Key Advantages of the KRAB System

- **Real-time situational awareness** of enemy UAV types and deployment areas.

- **Remote control and adjustment** of counter-UAV measures.

- **Close coordination** with own UAV units to avoid interference and warn reconnaissance UAVs of threats.

- **Timely warning** of enemy UAV threats to army units.

- **Identification of enemy drone launch points** and rapid coordination for artillery and strike UAV response.

---

### Future Development Prospects

- Integration of advanced detection and jamming equipment, including video receivers and transmitters at 2.4 GHz, 3.5 GHz.

- Deployment of higher power and frequency-selective jamming tools.

- Expansion of the KRAB system across all formations and units of the "Dnepr" grouping.

- Real-time data streaming to army command posts for centralized control.

---

### Conclusions

The **KRAB system** represents a significant advancement in comprehensive counter-UAV capabilities for the 49th Army. It has markedly increased detection and suppression efficiency against enemy commercial FPV drones and other reconnaissance UAVs, enabling coordinated, multi-level defense and timely offensive responses. The system’s modular architecture and integration with existing army EW, air defense, artillery, and UAV assets provide a robust model for future enhancement and wider deployment within the armed forces.